Application of th eigenvalues

Obtain the state equations for the following circuit

The node equation  I1=I2+I3                                              (3)

C2

R1

L

VS

I1

C3

R3

I2

I3

From the first loop  –VS+R1I1+VL+VC2=0        (1)

From the second  loop  –Vc2+Vc3  + R3 I3 =0  (2)

The node equation  I1=I2+I3                                              (3)

From (3)   I3= I1-I2

From (2)    –Vc2 + Vc3  +  R3 (I1-I2) =  0

–Vc2 + Vc3 +  R3 I1- R3 I2  = 0

–Vc2 + Vc3 +  R3 I1 =  R3 I2

Now replace I2 = CdVc2dt

–Vc2 + Vc3  +  R3 I1 =  R3 CdVc2dt

dVc2dt= −1R3C2Vc2+1R3C2Vc3+1R3C2R3I1   (E1)

From 1     VL=Vs-R1I1-Vc2

Replace   VL= L dILdt=dI1dt

L dI1dt=Vs-R1I1-Vc2

dI1dt=1LVs - 1LR1 I1- 1LVc2           (E2)

From 2     R3 I3 = Vc2-Vc3

Replace I3= C3dvc3dt

From 2    R3 C3dvc3dt = Vc2-Vc3

dvc3dt =1R3 C3 Vc2  -  1R3 C3 Vc3           (E3)

Write (E1)  (E2) (E3) in matrix form

dVc2dtdI1dt

This is  linear  differential equations.Homojen cozum icin kaynaklar sifir alinir. Yani Vs=0.

Problem AE-794 Calculate VC2(t), I1(t), VC3(t)in the above circuit if  Vs=0, R3=10,  C2=0.1,  L=0.1,  R1=1,  C3=0.1.  Sketch the graph of  VC2(t), I1(t), VC3(t) if  VC2(0)=200V      , I1(0)=-10A      ,   VC3(0)=20V

# Solution  Replace  R3=10,  C2=0.1,  L=0.1,  R1=1,  C3=0.1  in the above equation.

dVc2dtdI1dt

Eigenvalues and eigenvectors of A matrix are

λ1=-0.9,   λ2=-5.54 + 8.89i   λ2=-5.54 + 8.89i

X1=8.9-9.8, X2= 6.4 - 2.8i-0.3+7i ,  X3= 6.4 + 2.8i-0.3−7i

Thus the solution of the circuit is

VC2ILor

VC2(t)=C1 8.9 e-0.9t  + C2 (6.4-2.8i) e (-5.54+8.9i) t

+ C3 (6.4+2.8 i) e(-5.54-8.9i)t

IL(t)=-C1 9.8 e-0.9t  +(-0.3+7.07i) C2 e (-5.54+8.9i) t

+(-0.3-7.07i) C3  e (-5.54-8.9i)t

VC3(t)=C1 99 e-0.9t  + C2 (-0.5-0.4i) e (-5.54+8.9i) t

+ C3 (-0.5+0.4i) e (-5.54-8.9)t

replace t=0

200=C1 8.9  + C2 (6.4-2.8i)   + C3 (6.4+2.8 i)

-10=-C1 9.8 + C2 (-0.3+7.07i)+(-0.3-7.07i) C3

20=C1 99 + C2 (-0.5-0.4i)+ C3 (-0.5+0.4i)

8.9     6.4-2.8i          6.4+2.8i 9.8    -0.3+7.07i     -0.3-7.07i

The solution is

C1=0.35

C2=15.24 + 0.31i

C3=15.24 - 0.31i

Thus  the solution is

VC2(t)=3.11 e-0.9t  + (98–40i) e (-5.54+8.9i) t

+(98+40i) e(-5.54-8.9i)t

IL(t)=-3.43 e-0.9t  +(-6.7 +107i) e (-5.54+8.9i) t

+(-6.7 -107i) e (-5.54-8.9i)t

VC3(t)=34.6e-0.9t  + (-7.5-6.25i) e (-5.54+8.9i) t

+ (-7.5+6.25i) e (-5.54-8.9)t

Or if we write in sinusoid al form as in

Example AE-782

VC2(t)=3.11 e-0.9t  + 196 cos(8.9t) +80 sin(8.9t)

IL(t)=-3.43 e-0.9t  -13.4 cos(8.9t) -214 sin(8.9t)

VC3(t)=34.6e-0.9t  -15 cos(8.9t) +12.5 sin(8.9t)